### TLEN5370 - Lab 0

Lab 0 - no deliverable (pass/fail graded based on completion)

Objective - Familiarize yourself with lab procedures and equipment

- Plan your network

- Build your network

- Router interface configurations and show commands

- Static routing

- Saving and restoring configurations

- Cleanup

Useful Cisco Commands

- ? - shows all possible commands

- enable - enter enabled mode

- write erase - erases the configuration

- reload - reboots the router

- show ip interface brief - shows all interfaces

- show running-config - show the configuration

- show interface (name)(number) - show the interface

- write mem - writes the configuration

- ping x.x.x.x - pings the indicated IP address

- configure terminal - enter configuration mode

Objective 1 - Sit down with your group and plan your network

- Build a network topology of your choice with the following constraints:

  - Use four or more routers

  - Use at least two ethernet links

  - Use at least two serial links

  - Make sure at least one router does not directly connect to one other router

  - Connect your workstation into the network via ethernet so you can telnet to routers

  - Use 10/8 as your address space

Objective 2 - Build your network

- Determine the routers you will use, the interfaces and the cables. Connect them.

Objective 3 - Configure your network

- Login to the workstation.

- Use the workstations console cable via patch panels to access routers and set intial configurations.

- Start by erasing the old config "wr erase" followed by "reload" Don't use the autoconfig builder when it reboots.

- A few config commands that might be useful at the start are:

  - enable password (password) - this will set the password

  - ip subnet-zero - this allows you to use the 1st and last subnets...don't ask...its a remnant of pre-CIDR days

  - ip classless - this allows CIDR...for some reason it is off by default

  - line vty 0 4 followed by password (password) - this will allow you to telnet in from your workstation

  - hostname (routername) where routername is the name you give the router (something systematic might be useful especially when using m ultiple routers) - this sets the text of the prompt

  - You will lose points on future labs if you ask a question that is resolved by any of the above commands.

- From there build the interface configs.

- Once initial config is created use telnet to access the routers. (you might need to change IP of your workstation)

- Create the base config and verify you can ping between adjacent routers.

- Run some show commands to verify all is working correctly

Objective 4 - Configure some static routes and loopbacks

- Are your two non-touching routers able to reach one another?

- Use static routes to solve this problem.

- Verify all routers can reach all other routers

- Add a loopback on each router

- User static routes to make the loopbacks reachable from all other routers

Objective 5 - Saving, restoring, and documenting

- Determine a process for saving and then restoring configs. Remember most of the labs are multi-day and you will have to resume where you left off the next class.

- Also learn to carefully document all of your physical connections so you can recable your network quickly at the start of class.

Objective 6 - Cleanup

- Ever seen Jose mad? I'd recommend you do the following:

  - Coil all cables and return to bins

  - "wr erase" "reload" all routers

  - Clean up your workstation area